Site inspection report

The site inspection focuses on the implementation of risk controls used by contractors following the work health and safety (WHS) management plan assessment. The intent of the site inspection is to benchmark the safety practices observed on site against the criteria contained in the checklist. The Office of Industrial Relations understands that construction sites are continuously changing and vary greatly in size and complexity. For this reason, it may not be possible for the accredited auditor (construction) to examine all plant and work practices for a particular project. It is acceptable to review a sample of items and, based on the evidence that is available at the time, assess whether the undertaking conforms to the set criteria.

Principal contractor details

| Trading name |  | ABN |  |  |
| --- | --- | --- | --- | --- |
| Legal name |  |  |  |  |
| Business address |  |  |  |  |
| Contact person |  |  |  |  |
| Phone |  | Email |  |  |

Site representative(s) details (Duplicate this table if details are required of additional site representatives.)

| Name |  |
| --- | --- |
| Position |  |
| Level of involvement in the project |  |

Project details

| Project name |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Scope of work |  |  |  |  |  |  |
| Type of work |  Civil |  Residential |  |  Commercial |  |  Other (specify) |
| Project reference # |  |  | Job # |  |  |  |
| Contract duration (in weeks) |  |  | Acceptance date |  |  |  |
| Site address |  |  |  |  |  |  |
| Date of inspection |  |  | Inspection # |  |  |  |
| Contract superintendent |  |  |  |  |  |  |
| Phone |  |  | Email |  |  |  |

Auditor details

| Auditor name |  | Accreditation # |  |
| --- | --- | --- | --- |
| Phone |  | Email |  |
| Signature |  | Date |  |

Assessment result

| Overall result (Pass/Fail) |  | Total demerits |  |
| --- | --- | --- | --- |
| Non-compliance summary |  |  |  |

Ratings

	Conformance

	Non-conformance

U/V	Unable to be verified

N/A	Not applicable

Acknowledgement

<Insert Auditor Name>, accredited auditor (construction), acknowledges the cooperation and assistance provided by the staff and employees of <Insert Contractor Name> and thanks them for their contribution to this site inspection.

Confidentiality

Information obtained from <Insert Contractor Name> to assist in the development of this site inspection report will remain confidential and will only be used for purposes associated with the Prequalification (PQC) System.

Disclaimer and distribution

This report has been prepared by <Insert Auditor Name> for the sole purpose of assessing the standard of the health and safety management of <Insert Contractor Name>. While every effort has been made to ensure the accuracy of this report, <Insert Auditor Name> will not be held liable for the content and the consequences of its use. This report has been provided to the following:

The Contractor

The Contract Superintendent/Principal’s representative (PQC only)

The WHSQ safety program administrator (whsadmin@oir.qld.gov.au)

The PQC Registrar (pqcregistrar@hpw.qld.gov.au)

Summary

Introduction

As part of the Queensland Government pre-qualification requirements for building and civil construction contractors, <Insert Auditor Name> received a formal request by <Insert Contractor Name> to undertake a site inspection. This inspection focused on verifying that the safety practices undertaken on site were consistent with those contained in the WHS management plan. The inspection was undertaken on <Insert Audit Date> at <Insert Site Address>.

Inspection procedure

This inspection was conducted in accordance with the procedures outlined in the document Guidelines for accredited auditors (construction). The criteria used to benchmark the site safety practices of <Insert Contractor Name> are documented throughout the contents of this report. These criteria represent minimum occupational health and safety standards drawn from the Work Health and Safety Act 2011, the Work Health and Safety Regulation 2011, Queensland codes of practice, and relevant standards.

Record of inspection

This report contains a summary of the inspection findings. Details of documentation reviewed, people interviewed and other observations are recorded in the auditor’s notes and on the site inspection documentation. The auditor notes have been filed by <Insert Auditor Name> and will be made available upon request.

Findings

The key findings are summarised in the tables below.

Category 1:	A hazard of workplace activity likely to cause death, serious injury or serious illness

Category 2:	A hazard or workplace activity likely to cause other than death, serious injury or serious illness

Category 3:	Contravention of administrative requirements e.g. work plans, inductions, record keeping systems

| Risk category | Non-conformances |  | Demerits |
| --- | --- | --- | --- |
| Category 1 |  | x 25 demerits |  |
| Category 2 |  | x 2 demerit |  |
| Category 3 |  | x 1 demerit |  |
| Total |  |  |  |

Refer to the attached corrective action notices for further details.

Conclusion

From the information made available during the site inspection, <Insert Contractor Name> has been found to <conform/not conform> to the benchmarked OHS criteria, with XX demerit points issued.

A further site inspection <is/is not> required.

Note: Total demerit points ≥ 25 constitutes a major non-conformance and failed assessment.

Site inspection checklist index

1. Administrative requirements	5

2. Training, risk management, and general requirements	6

3. Consultation, cooperation, and coordination	7

4. Safe work method statements	7

5. The work environment	8

6. Confined spaces	12

7. Falling objects and public protection	14

8. Falls	17

9. Ladders and platforms supported by ladders	22

10. Scaffolding	24

11. Demolition	26

12. Welding and hot work	27

13. Essential underground services	29

14. Excavation	30

15. Electrical	30

16. Plant	31

17. Tilt-up and pre-cast concrete construction	37

18. Manual tasks	40

19. Hazardous substances and dangerous goods	41

20. Asbestos management	42

21. Noise	44

22. General traffic management	44

23. Hazardous atmosphere	44

25. Other hazards and risks	45

Site inspection checklist

#### 1. Administrative requirements

| Ref | Criteria | Observations | Rating | Demerits | CAN details |
| --- | --- | --- | --- | --- | --- |
| 1.1 | Signs have been installed that: - show the principal contractor’s name and telephone contact numbers (including an after hours telephone number) - show the location of the site office for the project, if any - are clearly visible from outside the workplace, or the work area of the workplace, where the construction project is being undertaken. Work Health and Safety Regulation 2011 s.308 |  |  |  |  |
| 1.2 | A written work health and safety management plan for the construction project was prepared prior to work commencing. Work Health and Safety Regulation 2011 s.309 |  |  |  |  |
| 1.3 | So far as reasonably practicable, all workers carrying out construction work on the project have been made aware of the content of the WHS management plan, including any updates, and their right to inspect the plan. Work Health and Safety Regulation 2011 s.310 and s.311 |  |  |  |  |
| 1.4 | The WHS management plan has been reviewed and, if necessary, revised to ensure it is up-to-date. Work Health and Safety Regulation 2011 s.311 |  |  |  |  |
| 1.5 | A copy of the WHS management plan has been kept by the principal contractor and is readily available for inspection by the auditor and by any worker carrying out construction work on the project. Work Health and Safety Regulation 2011 s.313 |  |  |  |  |

#### 2. Training, risk management, and general requirements

| Ref | Criteria | Observations | Rating | Demerits | CAN details |
| --- | --- | --- | --- | --- | --- |
| 2.1 | All workers carrying out construction work have received the general construction induction training. Work Health and Safety Regulation 2011 s316 |  |  |  |  |
| 2.2 | Workers have been informed of the site-specific health and safety rules. Work Health and Safety Regulation 2011 s.310 |  |  |  |  |
| 2.3 | Suitable and adequate information, training, and instruction has been provided to workers in relation to work they are carrying out. Work Health and Safety Regulation 2011 s.39 |  |  |  |  |
| 2.4 | Control measures have been maintained and reviewed to ensure they remain effective, fit for purpose, suitable, and set-up and used correctly. Work Health and Safety Regulation 2011 s.37 and s.38 |  |  |  |  |
| 2.5 | Personal protective equipment (PPE) has been provided to workers where it is prescribed as a control measure. Work Health and Safety Regulation 2011 s.44 |  |  |  |  |
| 2.6 | PPE is: - suitable for the relevant hazards - of suitable size and fit - maintained such that it is clean and hygienic and in good working order. Work Health and Safety Regulation 2011 s.44 |  |  |  |  |
| 2.7 | Workers have been provided with information, training, and instruction on the proper use and wearing of PPE. Work Health and Safety Regulation 2011 s.44 |  |  |  |  |
| 2.8 | Where it is prescribed as a control measure, PPE is being used by workers. Work Health and Safety Regulation 2011 s.44 |  |  |  |  |

#### 3. Consultation, cooperation, and coordination

| Ref | Criteria | Observations | Rating | Demerits | CAN details |
| --- | --- | --- | --- | --- | --- |
| 3.1 | Consultation is carried out with workers in accordance with the Act and the WHS management plan. Work Health and Safety Act 2011 s.47, s.48, and s.49 Work Health and Safety Regulation 2011 s.309 |  |  |  |  |
| 3.2 | Consultation, cooperation, and coordination with other duty holders on site is carried out in accordance with the WHS management plan. Work Health and Safety Act 2011 s.46 Work Health and Safety Regulation 2011 s.309 |  |  |  |  |
| 3.3 | Work groups have been formed in negotiation between workers and the Person Conducting the Business or Undertaking (PCBU), and health and safety representatives have been duly elected. Work Health and Safety Act 2011 s.50 and s.51 |  |  |  |  |
| 3.4 | Health and safety representatives (HSRs) have been involved in consultation. Work Health and Safety Act 2011 s.48 |  |  |  |  |

#### 4. Safe work method statements

| Ref | Criteria | Observations | Rating | Demerits | CAN details |
| --- | --- | --- | --- | --- | --- |
| 4.1 | A safe work method statement (SWMS) has been prepared for each high risk construction work activity that is underway or about to commence. Work Health and Safety Regulation 2011 s.299(1) |  |  |  |  |
| 4.2 | Each SWMS identifies the high risk construction work to which it applies. Work Health and Safety Regulation 2011 s.299(2)(a) |  |  |  |  |
| 4.3 | Each SWMS states the hazards relating to the high risk construction work and risks to health and safety associated with those hazards. Work Health and Safety Regulation 2011 s.299(2)(b) |  |  |  |  |
| 4.4 | Each SWMS describes the control measures to be implemented. Work Health and Safety Regulation 2011 s.299(2)(c) |  |  |  |  |
| 4.5 | Each SWMS describes how control measures are to be implemented, monitored, and reviewed. Work Health and Safety Regulation 2011 s.299(2)(d) |  |  |  |  |
| 4.6 | The relevant workers and HSRs were consulted in the preparation of each SWMS. Work Health and Safety Act 2011 s.49 |  |  |  |  |
| 4.7 | Each SWMS has been set out and expressed in a way that is readily accessible and understandable to the workers that are or will be required to use it. Work Health and Safety Regulation 2011 s.299(3)(b) |  |  |  |  |
| 4.8 | High risk construction work activities are being carried out in accordance with the relevant SWMS. Work Health and Safety Regulation 2011 s.300 |  |  |  |  |
| 4.9 | Where control measures have been revised, the relevant SWMS has been reviewed and, if necessary, revised. Work Health and Safety Regulation 2011 s.302 |  |  |  |  |
| 4.10 | A copy of each SWMS has been kept by the principal contractor for the required period and is readily available for inspection by the auditor and by any worker carrying out the relevant high risk construction work on the project. Work Health and Safety Regulation 2011 s.313 |  |  |  |  |

#### 5. The work environment

| Ref | Criteria | Observations | Rating | Demerits | CAN details |
| --- | --- | --- | --- | --- | --- |
| 5.1 | The work environment has been maintained such that it is clean and in a safe condition. Managing the Work Environment and Facilities Code of Practice 2011, 1.3 Maintaining the work environment and facilities |  |  |  |  |
| 5.2 | Work areas are clearly identified and separated as necessary so that work can be undertaken safely. Work Health and Safety Regulation 2011 s.40 |  |  |  |  |
| 5.3 | Floors and other surfaces are designed, installed, and maintained to allow work to be carried out without risk to health and safety. Work Health and Safety Regulation 2011 s.40 |  |  |  |  |
| 5.4 | Lighting at the workplace is sufficient to allow: - each worker to carry out work without risk to health and safety - people to move within the workplace without risk to health and safety - safe evacuation in an emergency. Work Health and Safety Regulation 2011 s.40 Managing the Work Environment and Facilities Code of Practice 2011, 2.6 Lighting |  |  |  |  |
| 5.5 | The workplace has sufficient ventilation to enable workers to carry out work without risk to health and safety. Work Health and Safety Regulation 2011 s.40 Managing the Work Environment and Facilities Code of Practice 2011, 2.7 Air quality |  |  |  |  |
| 5.6 | Workers carrying out work in extremes of heat or cold are able to carry out work without risk to health and safety. Work Health and Safety Regulation 2011 s.40 Managing the Work Environment and Facilities Code of Practice 2011, 2.8 Heat and cold |  |  |  |  |
| 5.7 | Outdoor workers have been provided with protection against adverse weather conditions and solar ultraviolet exposure. Work Health and Safety Regulation 2011 s.40 |  |  |  |  |
| 5.8 | Remote or isolated workers are provided with a system of work that includes effective communication for accessing assistance. Work Health and Safety Regulation 2011 s.48 |  |  |  |  |
| Housekeeping |  |  |  |  |  |
| 5.9 | The layout of the workplace allows, and the workplace is maintained so as to allow, for people to enter and exit and to move about without risk to health and safety, both under normal working conditions and in an emergency. Work Health and Safety Regulation 2011 s.40 |  |  |  |  |
| 5.10 | The principal contractor has managed the risks to health and safety associated with the storage, movement, and disposal of construction materials and waste at the workplace. Work Health and Safety Regulation 2011 s.315 |  |  |  |  |
| 5.11 | The entries, exits, and access ways in the workplace are kept clean and clear of materials and waste. Work Health and Safety Regulation 2011 s.40 |  |  |  |  |
| 5.12 | A safe system has been implemented for the collection, storage, and disposal of excess or waste materials. Work Health and Safety Regulation 2011 s.40 |  |  |  |  |
| 5.13 | Sufficient area has been allocated to safely store materials or plant for the construction work. Work Health and Safety Regulation 2011 s.40 and s.315 |  |  |  |  |
| 5.14 | Protruding objects that are a hazard have been made safe (e.g. exposed nails, vertical reinforcing steel). Work Health and Safety Regulation 2011 s.40 |  |  |  |  |
| Amenities |  |  |  |  |  |
| 5.15 | Adequate facilities have been provided for workers, including toilets, drinking water, washing facilities, and eating facilities. Work Health and Safety Regulation 2011 s.41 |  |  |  |  |
| 5.16 | Facilities provided for workers are in good working order and are clean, safe, and accessible. Work Health and Safety Regulation 2011 s.41 |  |  |  |  |
| 5.17 | Amenities are maintained in a hygienic, safe, and serviceable condition through a system for inspecting and cleaning the amenities. Work Health and Safety Regulation 2011 s.315A |  |  |  |  |
| 5.18 | At least one toilet has been provided for each 15, or part of 15, construction persons. Work Health and Safety Regulation 2011 Schedule 5A, Part 2 Toilets |  |  |  |  |
| 5.19 | Toilets are provided in accordance with the regulations, including the requirements for: - the accessibility of toilets for multi-story structures of at least four levels - whether toilets should be connected or portable - privacy, ventilation, and toilet paper - specific facilities for the hygienic disposal of sanitary items for females. Work Health and Safety Regulation 2011 s.315A and Schedule 5A, Part 2 Toilets |  |  |  |  |
| 5.20 | A room or sheltered area to eat meals and take breaks in, is reasonably available to each construction person and complies with the requirements relating to: - hygiene - separation from work activities that expose or are likely to expose construction workers to a health or safety risk - adequate space and seating - appropriate facilities for washing and storing utensils, boiling water, and storing food in a cool place. Work Health and Safety Regulation 2011 Schedule 5A, Part 3 Other amenities |  |  |  |  |
| 5.21 | Adequate clean water for washing the hands and face supplied through an appropriate facility are reasonably available to each construction person. Work Health and Safety Regulation 2011 Schedule 5A, Part 3 Other amenities |  |  |  |  |
| 5.22 | An appropriate and adequate supply of potable, clean, and cool drinking water is reasonably available to each construction person. Work Health and Safety Regulation 2011 Schedule 5A, Part 3 Other amenities |  |  |  |  |

| First aid |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 5.23 | First aid equipment has been provided for the workplace and is accessible to each worker. Work Health and Safety Regulation 2011 s.42 |  |  |  |  |
| 5.24 | An adequate number of workers have been trained to administer first aid. Work Health and Safety Regulation 2011 s.42 |  |  |  |  |
| 5.25 | The first aid provisions for the workplace are appropriate to the: - nature of the work being carried out - nature of the hazards at the workplace - size and location of the workplace - number and composition of the workers and other people at the workplace. Work Health and Safety Regulation 2011 s.42 |  |  |  |  |
| Site security at schools or similar facility |  |  |  |  |  |
| 5.26 | Where the workplace is located within an existing workplace such as a school or similar facility, the plan specifies how unauthorised access of children and staff will be prevented, including how the PCBU will communicate with the school or facility to manage this issue. Work Health and Safety regulation 2011 s.298 |  |  |  |  |

#### 6. Confined spaces

| Ref | Criteria | Observations | Rating | Demerits | CAN details |
| --- | --- | --- | --- | --- | --- |
| 6.1 | A SWMS is available for all construction work carried out in or near a confined space that has commenced or is about to commence. Work Health and Safety Regulation 2011 s.291 and s.299 |  |  |  |  |
| 6.2 | All confined spaces have been identified. Work Health and Safety Regulation 2011 Schedule 19, Dictionary - Confined Spaces Code of Practice, 1.4 How to determine whether a space is a confined space |  |  |  |  |
| 6.3 | All hazards associated with confined spaces have been identified. Confined Spaces Code of Practice 2011, 3. How to identify the hazards |  |  |  |  |
| 6.4 | For each hazard associated with a confined space, a competent person has carried out a documented risk assessment. Work Health and Safety Regulation 2011 s.66 |  |  |  |  |
| 6.5 | Risk assessments for confined spaces hazards are reviewed by a competent person to reflect any review and revision of control measures. Work Health and Safety Regulation 2011 s.66 |  |  |  |  |
| 6.6 | Workers that have been or are about to be directed to enter a confined space to carry out work hold a confined spaces entry permit in accordance with the regulations. Work Health and Safety Regulation 2011 s.67 |  |  |  |  |
| 6.7 | Appropriate signage has been erected for all confined spaces. Work Health and Safety Regulation 2011 s.68 |  |  |  |  |
| 6.8 | For all workers required to enter a confined space there is a system of work implemented that ensures: - continuous communication with the workers from outside the space - monitoring of conditions within the space by a standby person who is in the vicinity of the space and, if practicable, observing the work being carried out. Work Health and Safety Regulation 2011 s.69 |  |  |  |  |
| 6.9 | Any risks associated with the introduction of substances or condition into the confined space from or by any plant or services connected to the space have been identified, assessed, and controlled. Work Health and Safety Regulation 2011 s.70 |  |  |  |  |
| 6.10 | Any risks associated with the activation or energising in any way of any plant or services connected to the confined space have been identified, assessed, and controlled. Work Health and Safety Regulation 2011 s.70 |  |  |  |  |
| 6.11 | Atmospheric testing and monitoring is being carried out in all confined spaces by a competent person using suitable, correctly calibrated gas detectors. Confined Spaces Code of Practice 2011, 4. How to assess the risks |  |  |  |  |
| 6.12 | While work is being carried out in a confined space a safe oxygen level exists. If it is not reasonably practicable to ensure a safe oxygen level, workers carrying out work in the confined space have been provided with air supplied respiratory equipment. Work Health and Safety Regulation 2011 s.71 |  |  |  |  |
| 6.13 | Unauthorised entry into confined spaces has been prevented using locks, fixed barriers, or other security devices. Confined Spaces Code of Practice 2011, 5.9 Signs and barricades |  |  |  |  |
| 6.14 | If relevant, the specific controls associated with the atmosphere, flammable gases and vapours, and ignition sources within a confined space have been complied with. Work Health and Safety Regulation 2011 s.71, s.72, and s.73 |  |  |  |  |
| 6.15 | First aid procedures and rescue procedures to be followed in the event of an emergency in a confined space have been developed and practiced to ensure they are efficient and effective. Work Health and Safety Regulation 2011 s.74 |  |  |  |  |
| 6.16 | The entry and exit openings of all confined spaces are large enough to allow emergency access and are not obstructed. Work Health and Safety Regulation 2011 s.74 |  |  |  |  |
| 6.17 | Plant, equipment, and PPE provided for first aid or emergency rescue in relation to a confined space are maintained and in good working order. Work Health and Safety Regulation 2011 s.74 |  |  |  |  |
| 6.18 | PPE that is provided for emergencies relating to confined spaces is appropriate to the risks associated with each confined space. Work Health and Safety Regulation 2011 s.75 |  |  |  |  |
| 6.19 | Workers required to work in or around a confined space have been provided with information, instruction, and training in relation to the: - nature of all hazards relating to a confined space - need for, and the appropriate use of, control measures associated with those hazards - selection, fit, use, wearing, testing, storage, and maintenance of any personal protective equipment - contents of any confined space entry permits that may be issued - emergency procedures. Work Health and Safety Regulation 2011 s.76 |  |  |  |  |
| 6.20 | Records of all training provided to workers in relation to confined spaces is available and will be kept for a period of two years. Work Health and Safety Regulation 2011 s.76 |  |  |  |  |
| 6.21 | Copies of confined spaces risk assessments and confined spaces entry permits have been kept for the period required by the regulations. Work Health and Safety Regulation 2011 s.77 |  |  |  |  |

#### 7. Falling objects and public protection

| Ref | Criteria | Observations | Rating | Demerits | CAN details |
| --- | --- | --- | --- | --- | --- |
| 7.1 | Hoarding that has been used as a control measure: - will prevent any object that may reasonably be expected to hit it from entering the adjoining area - is strong enough, and appropriately designed and erected, for the circumstances in which it is used, including the location of the workplace and the type of work to be carried out near the hoarding. Work Health and Safety Regulation 2011 s.315E and s.315F |  |  |  |  |
| 7.2 | Perimeter containment screening that has been used as a control measure: - is made of mesh, timber, plywood or metal sheeting - has a gap of no more than 25mm between: - screens immediately next to each other - a screen and the framework supporting it - has a supporting framework that is able to bear the load of the screen. Work Health and Safety Regulation 2011 s.315I |  |  |  |  |
| 7.3 | Perimeter containment screening that has been used as a control measure complies with the regulations. Work Health and Safety Regulation 2011 s.315I |  |  |  |  |
| 7.4 | Where catch platforms that are used or are to be used as a control measure are installed, extended, or reduced, controls have been implemented to prevent a component of the platform falling on people while the platform is installed, extended, or reduced. Work Health and Safety Regulation 2011 s.315J |  |  |  |  |
| 7.5 | Gantries that have been used as a control measure have been designed by an engineer to withstand the required downward forces for the work being carried out. Work Health and Safety Regulation 2011 s.315K |  |  |  |  |
| 7.6 | Gantries that have been used as control measures: - are able to stop an object from falling that may reasonably be expected to fall on it - have an overhead platform that is secured to prevent it lifting or coming apart - have solid sheeting erected along the outer edge of its overhead platform to the required height - are able to stop water or dust falling on people - have natural or other lighting of at least 50 lux illuminating all of the area below it - will not tip over or rotate if a force that could reasonably be expected to be applied, is applied to it. Work Health and Safety Regulation 2011 s.315K |  |  |  |  |
| 7.7 | Any gantry that is being used to store materials or has had a shed erected on it has been designed by an engineer to take the additional load. Work Health and Safety Regulation 2011 s.315K |  |  |  |  |
| 7.8 | Written approval has been obtained from the relevant authority for the closure of any area. Work Health and Safety Regulation 2011 s.315M |  |  |  |  |
| 7.9 | Note: This criterion does not apply to civil or housing construction work. Barricading or hoarding that has been used as a control measure: - is of the required height - is constructed from appropriate materials - is correctly positioned with respect to the structure - must be able to withstand the relevant horizontal forces, if required by the regulations - must extend to the gantry’s overhead platform, if it is part of a gantry. Work Health and Safety Regulation 2011 s.315F |  |  |  |  |
| 7.10 | Note: This criterion does not apply to housing or civil construction work. Where a load is being lifted over an adjoining area: - the adjoining area has been closed at least to the extent necessary to prevent objects falling on or otherwise hitting people in the adjoining area, or - a gantry has been erected that provides adequate protection to people in the adjoining area against falling objects if the load were to fail. Work Health and Safety Regulation 2011 s.315L |  |  |  |  |
| 7.11 | Note: This criterion does not apply to demolition work or work erecting or dismantling formwork. Where the measured angle between the horizontal formed by an imaginary straight line drawn between: - the highest point at which work is being done on the structure - the point on the ground along the proposed line which any barricade or hoarding will be erected that is closest to the highest point is 75° or more, the additional control measures required under the regulations have been implemented. Work Health and Safety Regulation 2011 s.315G |  |  |  |  |

#### 8. Falls

| Ref | Criteria | Observations | Rating | Demerits | CAN details |
| --- | --- | --- | --- | --- | --- |
| 8.1 | A SWMS is available for all high risk construction work that involves a risk of a person falling more than 2m that has commenced or is about to commence. Work Health and Safety Regulation 2011 s.291 and s.299 |  |  |  |  |
| 8.2 | For all high risk construction work that involves a risk of a person falling more than 2m, where the only control measures to be implemented will be administrative or the provision of PPE, the SWMS describes all control measures considered in determining which control measure to implement (including by addressing the general fall protection requirements in section 79(3)). Work Health and Safety Regulation 2011 s.299 |  |  |  |  |
| 8.3 | Where a risk of fall of less than 3m in housing construction work or less than 2m in other construction work or construction work on roof with slope not over 26° exists: - each hazard that may result in a fall or cause death or injury if a person were to fall has been identified - the risk of death or injury that may result because of the hazard has been assessed - any control measures necessary to prevent, or minimise the level of, exposure to the risk is used. Work Health and Safety Regulation 2011 s.306C(3) | Examples of hazards: - Vertical reinforcing steel, or the edge of a rubbish skip, 1m below a surface from which the work is to be done. - Unsheeted floor bearers and joists 2m below a surface from which the work is to be done. - An object, for example a picket fence or stack of bricks, that could cause injury if a person fell on it. - A brittle roof on which the work is to be done 2m above a floor. |  |  |  |
| 8.4 | Where a risk of fall of at least 3m in housing construction work or at least 2m in other construction work or construction work on roof with a slope over 26° exists, control measures have been used which: - prevent a person falling any distance - if prevention is not practicable: - arrest a person’s fall - prevent or minimise the risk of death or injury to a person when the fall is arrested. Work Health and Safety Regulation 2011 s.306D(3) | Examples of control measures to prevent a person falling include: - edge protection - a fall protection cover placed over an opening - a travel restraint system. Examples of control measures to arrest a person’s fall include: - a fall arresting platform - a fall-arrest harness system - an industrial safety net. |  |  |  |
| Edge protection |  |  |  |  |  |
| 8.5 | Edge protection has been erected and is used in accordance with the instructions of: - the manufacturer or the supplier, where they have been provided, or - an engineer or competent person. Work Health and Safety Regulation 2011 s.306E |  |  |  |  |
| 8.6 | Edge protection has been designed to withstand the downwards or outwards force of the impact of a fall against it of any person who may reasonably be expected to fall against it. Work Health and Safety Regulation 2011 s.306E |  |  |  |  |
| 8.7 | Edge protection has: - a rail, or another component that prevents a person from falling, fitted in accordance with the regulations - another rail or rails or sturdy mesh, sheeting, or other material below the rail or component. Work Health and Safety Regulation 2011 s.306E |  |  |  |  |
| Fall protection covers |  |  |  |  |  |
| 8.8 | Each fall protection cover is able to withstand the impact of a fall onto it of any person who may reasonably be expected to fall onto it. Work Health and Safety Regulation 2011 s.306F |  |  |  |  |
| 8.9 | Each fall protection cover is securely fixed in place to prevent it being moved or removed accidentally. Work Health and Safety Regulation 2011 s.306F |  |  |  |  |
| Travel restraint systems |  |  |  |  |  |
| 8.10 | Travel restraint systems have been installed by a competent person. Work Health and Safety Regulation 2011 s.306G |  |  |  |  |
| 8.11 | Travel restraint systems have an anchorage point with a capacity to withstand any load that could be exerted on it in the normal operation of the system. Work Health and Safety Regulation 2011 s.306G |  |  |  |  |
| 8.12 | Travel restraint system users have been trained in the safe and correct use of the system. Work Health and Safety Regulation 2011 s.306G |  |  |  |  |
| 8.13 | Travel restraint systems in use do not show any evidence of wear or weakness that may affect the system’s safety. Work Health and Safety Regulation 2011 s.306G |  |  |  |  |
| 8.14 | Travel restraint systems have been inspected by a competent person at least every six months and a written record of the inspection exists. Work Health and Safety Regulation 2011 s.306G |  |  |  |  |
| Fall arrest platforms |  |  |  |  |  |
| 8.15 | Each fall arrest platform is able to withstand the impact of any person who may reasonably be expected to fall onto it. Work Health and Safety Regulation 2011 s.306H |  |  |  |  |
| 8.16 | Each fall arrest platform provides an unobstructed landing area for a falling person of at least 675mm wide for the length of the platform. Work Health and Safety Regulation 2011 s.306H |  |  |  |  |
| 8.17 | For each fall arrest platform where the slope of the surface from which construction work is to be carried out is not over 26°, the platform is not over 1m lower than the surface. Work Health and Safety Regulation 2011 s.306H |  |  |  |  |
| 8.18 | For each fall arrest platform where the slope of the surface from which construction work is to be carried out is over 26°, the platform is not over 300mm lower than the surface. Work Health and Safety Regulation 2011 s.306H |  |  |  |  |
| 8.19 | Each fall arrest platform has edge protection along the outer edge of the length and along each end of the platform that complies with the regulations. Work Health and Safety Regulation 2011 s.306H and s.306E |  |  |  |  |
| 8.20 | For each fall arrest platform where the gap between the inner edge of the length of the platform and the structure that it is immediately beside is greater than 225mm, controls to eliminate or minimise the risk of a person falling off the inner edge have been implemented. Work Health and Safety Regulation 2011 s.306H |  |  |  |  |
| Fall arrest harness systems |  |  |  |  |  |
| 8.21 | Each anchorage point of a fall arrest harness system has been: - designed by an engineer for that purpose - inspected and approved by a competent person prior to that point’s first use by any person. Work Health and Safety Regulation 2011 s.306I |  |  |  |  |
| 8.22 | Note: This criterion does not apply to anchorage points supporting a static line. Each anchorage point has sufficient capacity for the number of people using the anchorage point, as per the regulations. Work Health and Safety Regulation 2011 s.306I |  |  |  |  |
| 8.23 | Each fall arrest harness system’s device to absorb the energy of a falling person limits the force applied to the persons by a fall to not more than 6kN. Work Health and Safety Regulation 2011 s.306I |  |  |  |  |
| 8.24 | Each fall arrest harness system has been installed, maintained, and is used in accordance with: - the manufacturer or supplier, where they have been provided, or - an engineer or competent person. Work Health and Safety Regulation 2011 s.306I |  |  |  |  |
| 8.25 | Sufficient distance is available for a person using the system to fall to prevent the person hitting an object, the ground, or another surface other than a vertical surface. Work Health and Safety Regulation 2011 s.306I |  |  |  |  |
| 8.26 | No part of a fall arrest harness system can come into contact with anything that could affect the safe use of the system. Work Health and Safety Regulation 2011 s.306I |  |  |  |  |
| 8.27 | People using fall arrest harness system have been trained in the safe and correct use of the system. Work Health and Safety Regulation 2011 s.306I |  |  |  |  |
| 8.28 | Fall arrest harness systems show no evidence of wear or weakness to an extent that may affect the system’s safety. Work Health and Safety Regulation 2011 s.306I |  |  |  |  |
| 8.29 | Anchorage points that are considered to be worn or to have an impaired load bearing capacity are not used and appropriate measures have been taken to prevent their use. Work Health and Safety Regulation 2011 s.306I |  |  |  |  |
| 8.30 | Note: This criterion does not apply to anchorage points. Fall arrest harness systems are inspected by a competent person at least every six months and a written record of the inspection exists. Work Health and Safety Regulation 2011 s.306I |  |  |  |  |
| 8.31 | In the event that a fall arrest harness system has been used to arrest a fall, the system has been removed from use until its manufacturer or a competent person has inspected it and decided it is fit for purpose. Work Health and Safety Regulation 2011 s.306I |  |  |  |  |
| 8.32 | Fall arrest systems are not used by workers when they are alone. Work Health and Safety Regulation 2011 s.306I |  |  |  |  |
| 8.33 | Emergency and rescue procedures have been developed and tested. Work Health and Safety Regulation 2011 s.80 |  |  |  |  |
| 8.34 | Workers have been provided information, training, and instruction in relation to the emergency and rescue procedures. Work Health and Safety Regulation 2011 s.80 |  |  |  |  |
| Safety nets |  |  |  |  |  |
| 8.35 | Safety nets used as a control measure must: - be designed by an engineer or competent person for the purpose for which it is intended to be used - be made from a material designed to minimise injury to a person falling into the net - have energy absorbing characteristics to reduce the shock or injury to a person falling into the net. Work Health and Safety Regulation 2011 s.306J |  |  |  |  |
| 8.36 | Each safety net has been installed: - so that a person falling into the net will not hit anything below the net - as close as possible below the surface from which people might fall, but not more than the distance specified by the manufacturer or supplier or, where the manufacturer or supplier has not specified a distance, an engineer or competent person. Work Health and Safety Regulation 2011 s.306J |  |  |  |  |
| 8.37 | Each safety net has been installed in accordance with: - the manufacturer or supplier, where they have been provided, or - an engineer or competent person. Work Health and Safety Regulation 2011 s.306J |  |  |  |  |
| 8.38 | Each safety net is used, inspected, and maintained in accordance with: - the manufacturer or supplier, or - an engineer or competent person. Work Health and Safety Regulation 2011 s.306J |  |  |  |  |

#### 9. Ladders and platforms supported by ladders

| Ref | Criteria | Observations | Rating | Demerits | CAN details |
| --- | --- | --- | --- | --- | --- |
| Single or extension ladders – access only (i.e. no construction work or permitted work) |  |  |  |  |  |
| 9.1 | Note: This criterion does not apply to permitted work, as defined by s306A. Single or extension ladders are only used to gain access to a place (i.e. not to perform construction work). Work Health and Safety Regulation 2011 s.306K |  |  |  |  |
| 9.2 | When using a single or extension ladder to gain access to a place, workers maintain at least two hands and one foot, or two feet and one hand, on the ladder from when the person is fully on the ladder to when the person is leaving the ladder. Work Health and Safety Regulation 2011 s.306K |  |  |  |  |
| Single or extension ladders – construction work that is permitted work Note: The criteria under this heading only apply where a person performing the permitted work from a single or extension ladder could fall at least 3m for housing construction work, or at least 2m for all other construction work. |  |  |  |  |  |
| 9.3 | The person using the single or extension ladder: - has at least three limbs holding, wrapped around, or standing on the ladder in any combination, or is holding a stable object with one or both hands instead of the ladder, or - is prevented from falling by a control measure, or - is using a fall arrest harness system that is not attached to the ladder. Work Health and Safety Regulation 2011 s.306L |  |  |  |  |
| 9.4 | Each single or extension ladder is secured at the top or the bottom to prevent it moving. Work Health and Safety Regulation 2011 s.306L |  |  |  |  |
| 9.5 | A single or extension ladder used against a pole to do construction work has a device fitted at or near the top of the ladder between its sides that helps to ensure the ladder’s stability by partly accepting the shape of the pole. Work Health and Safety Regulation 2011 s.306L |  |  |  |  |
| Ladders generally Note: The criteria under this heading do not apply to trestle ladders. |  |  |  |  |  |
| 9.6 | Ladders have a load rating of at least 120kg and have been manufactured for industrial use. Work Health and Safety Regulation 2011 s.306M |  |  |  |  |
| 9.7 | Ladders are used only for the purpose for which they have been designed. Work Health and Safety Regulation 2011 s.306M |  |  |  |  |
| 9.8 | Ladders are not used to support a weight greater than that for which they have been designed. Work Health and Safety Regulation 2011 s.306M |  |  |  |  |
| 9.9 | Note: This criterion does not apply to ladders used in confined spaces. Ladders are no longer than: - 6.1m for single ladders - 9.2m for extension ladders used to do electrical work within the meaning of the Electrical Safety Act 2002 - 7.5m for other extension ladders. Work Health and Safety Regulation 2011 s.306M |  |  |  |  |
| 9.10 | Ladders are used where the: - bottom of the ladder is on a stable surface - rungs of the ladder are approximately level. Work Health and Safety Regulation 2011 s.306M |  |  |  |  |
| 9.11 | Ladders are not used to support a platform. Work Health and Safety Regulation 2011 s.306M |  |  |  |  |
| 9.12 | Single or extension ladders are only used at the appropriate angle, as per the regulations. Work Health and Safety Regulation 2011 s.306M |  |  |  |  |
| Work on platform supported by trestle ladders Note: The criteria below must be complied with before construction work is carried out on the platform supported by trestle ladders where the person performing the work could fall at least 3m, for housing construction work, or at least 2m for all other construction work. |  |  |  |  |  |
| 9.13 | Each trestle ladder is secured to prevent it from moving. Work Health and Safety Regulation 2011 s.306N |  |  |  |  |
| 9.14 | Edge protection that complies with the regulations has been erected along the outer edge of the length of the platform. Work Health and Safety Regulation 2011 s.306N |  |  |  |  |
| 9.15 | Control measures are in place to eliminate or minimise the risk of a person falling off the inner edge or the end of the platform. Work Health and Safety Regulation 2011 s.306N |  |  |  |  |
| Platform supported by trestle ladders |  |  |  |  |  |
| 9.16 | Platforms are unobstructed. Work Health and Safety Regulation 2011 s.306O |  |  |  |  |
| 9.17 | For construction work being carried out on a platform supported by trestle ladders where there is a risk that the person carrying out the work could fall less than 3m, for housing construction, or less than 2m for all other construction: - for light work, as defined by the regulations, the platform is at least 225mm wide along its entire length, or - for all other work the platform is at least 450mm wide along its entire length. Work Health and Safety Regulation 2011 s.306O |  |  |  |  |
| 9.18 | For construction work being carried out on a platform supported by trestle ladders where there is a risk that the person carrying out the work could fall at least 3m, for housing construction, or at least 2m for all other construction: - the platform is at least 450mm wide along its entire length - the platform is not higher than 5m. Work Health and Safety Regulation 2011 s.306O |  |  |  |  |

#### 10. Scaffolding

| Ref | Criteria | Observations | Rating | Demerits | CAN details |
| --- | --- | --- | --- | --- | --- |
| 10.1 | Scaffolding has been erected by appropriately licensed workers. Work Health and Safety Regulation 2011 s.81 and schedule 3 |  |  |  |  |
| 10.2 | Scaffolding is or has been erected and/or dismantled so as to eliminate or minimise the risk of falling at least 3m, in housing construction, or 2m in all other construction to people carrying out the work, in accordance with the regulations Work Health and Safety Regulation 2011 s.306P and s.306Q |  |  |  |  |
| 10.3 | Safe access to and egress from scaffold has been provided during erection, use, and dismantling. Scaffolding Code of Practice, 5.10 Access and egress |  |  |  |  |
| 10.4 | Written confirmation from a competent person stating that the construction of a particular scaffold has been completed, has been obtained prior to the use of that particular scaffold. Work Health and Safety Regulation 2011 s.225 |  |  |  |  |
| 10.5 | Each scaffold and its supporting structure has been inspected by a competent person: - before the scaffold is used - before use of the scaffold is resumed after an incident occurs that may reasonably be expected to affect the stability of the scaffold - before use of the scaffold is resumed after repairs - at least every 30 days. Work Health and Safety Regulation 2011 s.225 |  |  |  |  |
| 10.6 | Scaffold or supporting structures that have been identified by an inspection as creating a risk to health and safety are repaired, altered, and added to as necessary, and inspected again by a competent person prior to use. Work Health and Safety Regulation 2011 s.225 |  |  |  |  |
| 10.7 | Unauthorised access to the scaffold is prevented while the scaffold is incomplete or unattended. Work Health and Safety Regulation 2011 s.225 |  |  |  |  |
| 10.8 | A scaffold plan is available for each scaffold and complies with Scaffolding Code of Practice 2009, 3. Planning for scaffold work |  |  |  |  |
| 10.9 | Each scaffold has been erected, used, and dismantled in accordance with the: - scaffold plan - manufacturer’s instructions and specifications - Scaffolding Code of Practice 2009 - AS/NZS 1576:2010 Scaffolding. |  |  |  |  |

#### 11. Demolition

| Ref | Criteria | Observations | Rating | Demerits | CAN details |
| --- | --- | --- | --- | --- | --- |
| 11.1 | The regulator has been notified at least five days prior to any demolition work specified under this section. Work Health and Safety Regulation 2011 s.142 |  |  |  |  |
| 11.2 | A SWMS is available for all work that involves demolition of an element of a structure that is load-bearing or otherwise related to the physical integrity of the structure that has commenced or is about to commence. Work Health and Safety Regulation 2011 s.291 and s.299 |  |  |  |  |
| 11.3 | The risk of falling objects has been controlled in accordance with the regulations. Work Health and Safety Regulation 2011 s.315H |  |  |  |  |
| 11.4 | The PCBU performing the demolition work holds the appropriate licence and it is readily available for inspection. Work Health and Safety Regulation 2011 s.143 and s.144M |  |  |  |  |
| 11.5 | The nominated supervisor for the demolition work is readily available to workers when the demolition work is being carried out. Work Health and Safety Regulation 2011 s.144 |  |  |  |  |
| Demolition involving asbestos |  |  |  |  |  |
| 11.6 | Information about asbestos and asbestos containing materials (ACM) has been obtained and reviewed prior to demolition work being carried out. Work Health and Safety Regulation 2011 s.448 and s.450 |  |  |  |  |
| 11.7 | If no asbestos register exists, the structure or plant has been inspected by a competent person to determine whether asbestos or ACM exist. Work Health and Safety Regulation 2011 s,451 |  |  |  |  |
| 11.8 | Asbestos likely to be disturbed by the demolition has been identified and, so far is reasonably practicable, removed. Work Health and Safety Regulation 2011 s.452 |  |  |  |  |
| 11.9 | A procedure has been developed that reduces the risk of exposure of workers and other people to asbestos to below the exposure standard. Work Health and Safety Regulation 2011 s.454 |  |  |  |  |

#### 12. Welding and hot work

| Ref | Criteria | Observations | Rating | Demerits | CAN details |
| --- | --- | --- | --- | --- | --- |
| 12.1 | Workers are not exposed to welding fumes that contain a substance or mixture in an airborne concentration that exceeds the exposure standard for the substance or mixture. Work Health and Safety Regulation 2011 s.49 Welding processes Code of Practice 2013, 3.1 Airborne contaminants. |  |  |  |  |
| 12.2 | Non-flammable screens or partitions have been installed where appropriate. Work Health and Safety Act 2011 s.19(1) Welding processes Code of Practice 2013, 3.2 Radiation |  |  |  |  |
| 12.3 | Signs have been erected to warn workers and passers-by that welding is occurring and that entry into the work area is not permitted unless PPE is worn. Work Health and Safety Act 2011 s.19(1) Welding processes Code of Practice 2013, 3.2 Radiation |  |  |  |  |
| 12.4 | Appropriate PPE has been provided for and is being used by workers performing welding work i.e. filter shades for goggles and face shields to protect the eyes from radiation, gloves and other protective clothing to cover exposed skin. Work Health and Safety Act 2011 s.19(1) Welding processes Code of Practice 2013, 3.2 Radiation |  |  |  |  |
| 12.5 | The risk of electric shock from contact with the electrode, live parts, the work piece, or through contact with an unearthed cable, lead, or tool has been controlled. Work Health and Safety Act 2011 s.19(1) Welding processes Code of Practice 2013, 3.3 Electrical risks |  |  |  |  |
| 12.6 | Welding equipment is used with earth leakage current protection by means of a residual current device. Work Health and Safety Act 2011 s.19(1) Welding processes Code of Practice 2013, 3.3 Electrical risks |  |  |  |  |
| 12.7 | Risks to workers from equipment that generates strong electromagnetic fields have been identified and controlled. Work Health and Safety Act 2011 s.19(1) Welding processes Code of Practice 2013, 3.3 Electrical risks |  |  |  |  |
| 12.8 | All sources of ignition have been identified and isolated from fuel sources. Work Health and Safety Act 2011 s.19(1) Welding processes Code of Practice 2013, 3.4 Fire and explosion |  |  |  |  |
| 12.9 | Fire fighting equipment is located near the welding work area. Work Health and Safety Act 2011 s.19(1) Welding processes Code of Practice 2013, 3.4 Fire and explosion |  |  |  |  |
| 12.10 | Workers are isolated from contacting hot work pieces and otherwise protected from burns and exposure to heat. Work Health and Safety Act 2011 s.19(1) Welding processes Code of Practice 2013, 3.5 Burns and exposure |  |  |  |  |
| 12.11 | Compressed and liquefied gas cylinders are stored and handled in accordance with AS 4332-2004 – The storage and handling of gases in cylinder. Work Health and Safety Act 2011 s.19(1) Welding processes Code of Practice 2013, 3.6 Compressed and liquefied gases |  |  |  |  |
| 12.12 | Cylinders are stored in an upright position and have been secured to prevent dislodgement. Work Health and Safety Act 2011 s.19(1) Welding processes Code of Practice 2013, 3.6 Compressed and liquefied gases |  |  |  |  |
| 12.13 | Flashback arrestors on hoses have been installed and have been tested at intervals of at least every 12 months. Work Health and Safety Act 2011 s.19(1) Welding processes Code of Practice 2013, 3.6 Compressed and liquefied gases |  |  |  |  |
| 12.14 | Cylinder valves are closed when the cylinder is not being used. Work Health and Safety Act 2011 s19(1) Welding processes Code of Practice 2013, 3.6 Compressed and liquefied gases |  |  |  |  |
| 12.15 | Cylinders have been fitted with a bursting disc safety device or an operational spring-loaded pressure relief valve as appropriate. Work Health and Safety Act 2011 s.19(1) Welding processes Code of Practice 2013, 3.6 Compressed and liquefied gases |  |  |  |  |
| 12.16 | Work area is well ventilated and an air supplied respirator has been supplied when necessary. Work Health and Safety Act 2011 s.19(1) Welding processes Code of Practice 2013, 3.6 Compressed and liquefied gases |  |  |  |  |
| 12.17 | Cylinder fittings, hoses, and connections are not damaged or in poor condition. Work Health and Safety Act 2011 s.19(1) Welding processes Code of Practice 2013, 3.6 Compressed and liquefied gases |  |  |  |  |

#### 13. Essential underground services

| Ref | Criteria | Observations | Rating | Demerits | CAN details |
| --- | --- | --- | --- | --- | --- |
| 13.1 | The risks to health and safety associated with any essential services at the workplace have been identified and managed. Work Health and Safety Regulation 2011 s.315 |  |  |  |  |
| 13.2 | A SWMS is available for all work that has commenced or is about to commence that involves work on or near: - pressurised gas distribution mains or piping - chemical, fuel, or refrigerate lines - energised electrical installations or services. Work Health and Safety Regulation 2011 s.291 and s.299 |  |  |  |  |
| 13.3 | Current underground essential services information is available for inspection for all areas where excavation work is being carried out and for all adjacent areas. Work Health and Safety Regulation 2011 s.304 |  |  |  |  |
| 13.4 | Workers carrying out excavation work have been provided with the current underground essential services information. Work Health and Safety Regulation 2011 s.304 |  |  |  |  |

#### 14. Excavation

| Ref | Criteria | Observations | Rating | Demerits | CAN details |
| --- | --- | --- | --- | --- | --- |
| 14.1 | A SWMS is available for all excavation work that includes the use of powered mobile plant that has commenced or is about to commence. Work Health and Safety Regulation 2011 s.291 and s.299 |  |  |  |  |
| 14.2 | The risks to health and safety associated with any excavation work have been identified and managed, including the risk of a person: - falling into the excavation - being trapped by the collapse of an excavation - working in an excavation being struck by a falling object - working in an excavation being exposed to an airborne contaminant. Work Health and Safety Regulation 2011 s.305 |  |  |  |  |
| Trenching |  |  |  |  |  |
| 14.3 | For trenches 1.5m or more deep, the work area has been, so far as is reasonably practicable, secured from unauthorised access, including inadvertent entry. Work Health and Safety Regulation 2011 s.306 |  |  |  |  |
| 14.4 | For trenches 1.5m or more deep, the sides of the trench have been adequately supported by one of the following: - shoring by shielding or other comparable means - benching - battering. Note: This does not apply if PCBU has written advice, which complies with the regulations, from a geotechnical engineer that all sides of the trench are safe from collapse has been obtained. Work Health and Safety Regulation 2011 s.306 |  |  |  |  |

#### 15. Electrical

| Ref | Criteria | Observations | Rating | Demerits | CAN details |
| --- | --- | --- | --- | --- | --- |
| 15.1 | Work is conducted in a way that is electrically safe. Electrical Safety Act 2002 s.30 |  |  |  |  |
| 15.2 | A SWMS is available for all work that is carried out on or near energised electrical installations or services that has commenced or is about to commence at the workplace. Work Health and Safety Regulation 2011 s.291 and s.299 |  |  |  |  |
| 15.3 | Electrical leads are arranged to avoid damage. Electrical Safety Regulation 2013 s.102 |  |  |  |  |
| 15.4 | Leads and tools are not used in damp or wet conditions unless they have been specifically designed to do so. Electrical Safety Act 2002 s30 |  |  |  |  |
| 15.5 | Residual current devices are installed and tested regularly. Electrical Safety Regulation 2013 s.104 |  |  |  |  |
| 15.6 | Electrical equipment has been inspected, testing, and tagged in accordance with AS/NZS 3012 Electrical installations – Construction and demolition sites. |  |  |  |  |
| 15.7 | Electrical work is carried out by an appropriately licensed individual. Electrical Safety Act 2002 s.55 |  |  |  |  |
| 15.8 | All electrical equipment for the performance of work is in accordance with the requirements of AS/NZS 3012 Electrical installations – Construction and demolition sites. |  |  |  |  |
| 15.9 | The appropriate exclusion zones have been established and are enforced. Electrical Safety Regulations 2002, Schedule 2 Exclusion zones for electrical parts |  |  |  |  |

#### 16. Plant

Note: The criteria under this heading do not apply to plant that rely exclusively on manual power for operation and are designed to be primarily supported by hand (Work Health and Safety Regulation 2011 s185).

| Ref | Criteria | Observations | Rating | Demerits | CAN details |
| --- | --- | --- | --- | --- | --- |
| Plant and structures |  |  |  |  |  |
| 16.1 | The risks to health and safety associated with any plant at the workplace have been identified and managed. Work Health and Safety Act 2011 s.21 Work Health and Safety Regulation 2011 s.203 |  |  |  |  |
| 16.2 | Unauthorised alterations or interference with plant has been prevented. Work Health and Safety Regulation 2011 s.205 |  |  |  |  |
| 16.3 | Plant is used only for the purpose for which it has been designed or otherwise in a way that does not increase the risk to health and safety. Work Health and Safety Regulation 2011 s.206 |  |  |  |  |
| 16.4 | Health and safety features and warning devices, are operational and are used in accordance with the relevant instructions and information. Examples include: - guarding - operational controls - emergency stops - other warning devices. Work Health and Safety Regulation 2011 s.206 |  |  |  |  |
| 16.5 | Plant that is not in use has been left in a state that does not create a risk to the health and safety of any person. Work Health and Safety Regulation 2011 s.207 |  |  |  |  |
| 16.6 | The principal contractor has managed the risks to health and safety associated with the storage of workplace plant that is not in use. Work Health and Safety Regulation 2011 s.315 |  |  |  |  |
| 16.7 | Warning devices are positioned on the plant to ensure that the device will work to best effect. Work Health and Safety Regulation 2011 s.212 |  |  |  |  |
| Guarding |  |  |  |  |  |
| 16.8 | Guarding used as a control measure must: - be solidly constructed and mounted - make by-passing or disabling difficult - not create a risk to health and safety - be is properly maintained - control the risk of broken or ejected parts and workpieces - be removable to allow maintenance while plant is not in normal operation - be replaced before the plant can be restarted. Work Health and Safety Regulation 2011 s.208 |  |  |  |  |
| 16.9 | Pipes or other parts of plant associated with heat or cold are guarded or insulated. Work Health and Safety Regulation 2011 s.209 |  |  |  |  |
| Operator controls |  |  |  |  |  |
| 16.10 | Ensure that any operator’s controls are: - identified on the plant and indicates the nature, function and direction of operation - located to be readily accessible for convenient operation - located or guarded to prevent unintentional operation/activation - able to be locked in the off position. Work Health and Safety Regulation 2011 s.210 |  |  |  |  |
| Emergency stops |  |  |  |  |  |
| 16.11 | Where plant is operated by more than one person, the emergency stops are fitted and must: - be prominently, clearly and durably marked - be immediately accessible by the operator(s) - have a red coloured handle, bar or push button - not be adversely affected by electrical or electronic malfunction. Work Health and Safety Regulation 2011 s.211 |  |  |  |  |
| Maintenance and inspection |  |  |  |  |  |
| 16.12 | Maintenance, inspection, and testing is carried out by a competent person. Work Health and Safety Regulation 2011 s.213 |  |  |  |  |
| 16.13 | Maintenance, inspection, and testing is carried out: - in accordance with the manufacturer’s recommendations - where there are no manufacturer’s recommendations, in accordance with the recommendations of a competent person - annually. Work Health and Safety Regulation 2011 s.213 |  |  |  |  |
| 16.14 | Records of all tests, inspections, maintenance, commissioning, decommissioning, dismantling, and alternation of each item of plant are available. Managing Risk of Plant in the Workplace Code of Practice 2013, 6. Keeping Records. |  |  |  |  |
| Powered mobile plant |  |  |  |  |  |
| 16.15 | Assess risks associated with: - plant overturning - things falling on the operator - the operator being ejected - the plant colliding with any person or thing - mechanical failure of pressurised elements of the plant that may release fluids that pose a risk to health and safety have been identified and managed. Work Health and Safety Act 2011 s.21 Work Health and Safety Regulation 2011 s.214 |  |  |  |  |
| 16.16 | A SWMS is available for work that includes the use of powered mobile plant that has commenced or is about to commence. Work Health and Safety Regulation 2011 s.291 and s.299 |  |  |  |  |
| 16.17 | Mobile plant operators hold the appropriate high risk work licence or, for earthmoving equipment and particular cranes, have been deemed competent. Work Health and Safety Regulation 2011 s.81 and schedule |  |  |  |  |
| 16.18 | A suitable combination of operator protective devices for each item of powered mobile plant have been provided, maintained, and used. Work Health and Safety Regulation 2011 s.215 |  |  |  |  |
| 16.19 | People other than the operator of an item of mobile plant are prevented from riding on the plant or are otherwise provided with a level of protection that is equivalent to that provided to the operator. Work Health and Safety Regulation 2011 s215 |  |  |  |  |
| 16.20 | Powered mobile plant is prevented from colliding with pedestrians or other powered mobile plant. Work Health and Safety Regulation 2011 s.215 |  |  |  |  |
| 16.21 | Where there is a possibility of powered mobile plant colliding with pedestrians or other powered mobile plant, a warning device has been fitted. Work Health and Safety Regulation 2011 s.215 |  |  |  |  |
| 16.22 | Each industrial lift truck has been equipped with lifting attachments that are suitable for the load being lifted or moved. Work Health and Safety Regulation 2011 s.218 |  |  |  |  |
| 16.23 | Each industrial lift truck is operated in a manner that ensures risks to the operator or other nearby people are eliminated or minimised. Work Health and Safety Regulation 2011 s.218 |  |  |  |  |
| 16.24 | Industrial lift trucks are not used to carry passengers unless the: - truck is designed to carry a seated passenger - passenger seat is fitted with suitable restraints - passenger seat is located within the zone of protection that is provided by the operator protective device. Work Health and Safety Regulation 2011 s.218 |  |  |  |  |
| Plant that lifts or suspends loads |  |  |  |  |  |
| 16.25 | Operators of plant used to lift or suspend a load hold the appropriate high risk work licence. Work Health and Safety Regulation 2011 s.81 and schedule 3 |  |  |  |  |
| 16.26 | Plant used to lift or suspend a load has been specifically designed to lift or suspend the relevant load, or otherwise complies with the regulations. Work Health and Safety Regulation 2011 s.219 |  |  |  |  |
| 16.27 | Lifting and suspending is carried out: - with lifting attachments that are suitable for the load - within the safe working limits of the plant. Work Health and Safety Regulation 2011 s.219 |  |  |  |  |
| 16.28 | Loads are not suspended or travelled over a person unless the plant is specifically designed for that purpose. Work Health and Safety Regulation 2011 s.219 |  |  |  |  |
| 16.29 | Loads are lifted or suspended in a way that ensures that the load remains under control during the activity. Work Health and Safety Regulation 2011 s.219 |  |  |  |  |
| 16.30 | The following measures must be followed for plant that is used to lift or suspend a person, but has not been specifically designed to do so. - People are lifted or suspended in a work box that is securely attached to the plant. - Where there is a risk of a person falling from a height, each person wears a safety harness. - Means are provided by which the persons being lifted or suspended can safely exit the plant in the event of a failure. Work Health and Safety Regulation 2011 s.220 |  |  |  |  |
| Mobile and tower cranes |  |  |  |  |  |
| 16.31 | Mobile and tower crane operators hold the appropriate high risk work licence. Work Health and Safety Regulation 2011 s.81, schedule 3 |  |  |  |  |
| 16.32 | Maintenance, inspection, and testing of cranes is carried out by a competent person. Work Health and Safety Regulation 2011 s.213 and s.235 |  |  |  |  |
| 16.33 | Cranes must undergo a major inspection: - at the end of the design life recommended by the manufacturer - in accordance with the recommendations of a competent person if there are no manufacturer’s recommendations - every 10 years from the date the crane was first commissioned or first registered, whichever occurred first. Work Health and Safety Regulation 2011 s.235 |  |  |  |  |
| Registered plant (plant designs and plant items) |  |  |  |  |  |
| 16.34 | Each item of registrable plant, as per the regulations, has been registered. Work Health and Safety Regulation 2011 s.243, s.246 and schedule 5 parts 1 and 2 |  |  |  |  |
| 16.35 | A record of all tests, inspections, maintenance, commissioning, decommissioning, dismantling, and alteration of each item of registrable plant has been kept and is available for inspection. Work Health and Safety Regulation 2011 s.237 |  |  |  |  |
| Compliance with codes of practice and standards |  |  |  |  |  |
| 16.36 | Mobile cranes used in accordance with Mobile Crane Code of Practice 2006, AS/NZS 1418.5, AS/NZS 2550.1 and AS/NZS 2550.5. |  |  |  |  |
| 16.37 | Hoists used in accordance with AS/NZS 1418.7. |  |  |  |  |
| 16.38 | Explosive powered tools used in accordance with AS/NZS 1873.1. |  |  |  |  |
| 16.39 | Lasers used in accordance with AS/NZS 2397. |  |  |  |  |
| 16.40 | Fire extinguishers compliant with AS/NZS 1851. |  |  |  |  |

#### 17. Tilt-up and pre-cast concrete construction

| Ref | Criteria | Observations | Rating | Demerits | CAN details |
| --- | --- | --- | --- | --- | --- |
| 17.1 | A SWMS is available for all work that involves tilt-up or precast concrete that has commenced or is about to commence. Work Health and Safety Regulation 2011 s.291 and s.299 |  |  |  |  |
| 17.2 | Detailed drawings have been provided and have been checked by a suitably qualified engineer. Tilt-up and Pre-cast Construction Code of Practice 2003, 4 Drawings |  |  |  |  |
| 17.3 | Design certification from a professional engineer is available. Tilt-up and Pre-cast Construction Code of Practice 2003, 9.1 Engineer certification |  |  |  |  |
| 17.4 | A system is in place to monitor work on site to ensure compliance with designs and specifications. Tilt-up and Pre-cast Construction Code of Practice 2003, 9.2 On-site verification |  |  |  |  |
| 17.5 | Work is being carried out in accordance with the designs and specifications. Tilt-up and Pre-cast Construction Code of Practice 2003, 9.2 On-site verification |  |  |  |  |
| 17.6 | The appropriate type and number of bracing is being used and has been installed correctly. Tilt-up and Pre-cast Construction Code of Practice 2003, 5 Braces |  |  |  |  |
| 17.7 | Engineering certification is available for all bracing. Tilt-up and Pre-cast Construction Code of Practice 2003, 5 Braces, 6 Wind loading, 7 Deadman design, and 8 Drill-in type anchors |  |  |  |  |
| 17.8 | A lift plan has been prepared and includes: - crane set-up position - locations where panels are to be lifted from and to - obstacles - areas where cranes cannot be set-up - the type and set-up of lifting gear - responsibilities for rigging, dogging, and spotting duties. Tilt-up and Pre-cast Construction Code of Practice 2003, 12.1 Lift plan |  |  |  |  |
| 17.9 | Exclusion zones have been established and are enforced. Tilt-up and Pre-cast Construction Code of Practice 2003, 13.1 Exclusion zones for tilt-up panel erection |  |  |  |  |
| Risk assessment for panel erections |  |  |  |  |  |
| 17.10 | The risk assessment for panel erections has considered the ground slope and condition. Tilt-up and Pre-cast Construction Code of Practice 2003, 12.5 Ground slope and condition |  |  |  |  |
| 17.11 | The risk assessment for panel erections has considered the crane type and capacity. Tilt-up and Pre-cast Construction Code of Practice 2003, 12.2 Crane capacity, 12.3 ‘Pick and carry’ cranes, and 12.4 Slewing cranes on outriggers |  |  |  |  |
| 17.12 | The risk assessment for panel erections has considered the impact of wind loading. Tilt-up and Pre-cast Construction Code of Practice 2003, 6 Wind loading |  |  |  |  |
| 17.13 | The risks arising from lifting pre-cast elements over workers have been identified and managed. Tilt-up and Pre-cast Construction Code of Practice 2003, 13.2 Lifting pre-cast elements over workers |  |  |  |  |
| Concrete pumping |  |  |  |  |  |
| 17.14 | Concrete pump is located in the most favourable position to pump concrete, including allowing adequate visual contact for the pump operator with both the pump area and the pour area. Concrete Pumping Code of Practice 2005, 3.1 Planning by the principal contractor or person in control of the workplace |  |  |  |  |
| 17.15 | A clear, level area of ground with a firm base capable of supporting the pump unit and the concrete delivery trucks is available. Concrete Pumping Code of Practice 2005, 3.1 Planning by the principal contractor or person in control of the workplace |  |  |  |  |
| 17.16 | Clear access to the pump unit for concrete trucks exists. Concrete Pumping Code of Practice 2005, 3.1 Planning by the principal contractor or person in control of the workplace |  |  |  |  |
| 17.17 | A clearly defined ‘pump washout area’ complying with environmental protection legislation and local authority requirements exists. Concrete Pumping Code of Practice 2005, 3.1 Planning by the principal contractor or person in control of the workplace |  |  |  |  |
| 17.18 | A method to collect concrete residue and/or all necessary precautions to prevent wash down residue from the clean-up operations entering stormwater drains has been implemented. Concrete Pumping Code of Practice 2005, 3.1 Planning by the principal contractor or person in control of the workplace |  |  |  |  |
| 17.19 | Appropriate inspections and maintenance have been carried out for all concrete pumping equipment and written records are kept with each unit. Concrete Pumping Code of Practice 2005, 5 Inspection and maintenance |  |  |  |  |
| 17.20 | All equipment associated with the concrete pumping has a fixed compliance plate and/or appropriate certification for their use. Concrete Pumping Code of Practice 2005, 5.2 Compliance plates and certification |  |  |  |  |
| 17.21 | Concrete pipelines are appropriately positioned and restrained. Concrete Pumping Code of Practice 2005, 4.1.3 Concrete pipelines and restraint equipment |  |  |  |  |
| 17.22 | Work is undertaken in accordance with the Concrete Pumping Code of Practice 2005. |  |  |  |  |
| Formwork |  |  |  |  |  |
| 17.23 | The formwork system has been certified by: - a formwork designer, for basic formwork systems, or - an engineer, for non-basic formwork systems. Formwork of Practice 2006, 2.2.2 Formwork design certification requirements |  |  |  |  |
| 17.24 | The necessary formwork documentation, including formwork drawings, has been prepared and is available for inspection on site. Formwork Code of Practice 2006, 2.2.3 Documentation and 2.2.5 On-site coordination and verification |  |  |  |  |
| 17.25 | Work is carried out in accordance with the formwork system. Formwork Code of Practice 2006, 2.2.5 On-site coordination and verification |  |  |  |  |
| 17.26 | On-site inspections are carried out to ensure compliance with the formwork system. Formwork Code of Practice 2006, 2.2.5 On-site coordination and verification |  |  |  |  |
| 17.27 | Work is undertaken in accordance with the Formwork Code of Practice 2006. |  |  |  |  |

#### 18. Manual tasks

| Ref | Criteria | Observations | Rating | Demerits | CAN details |
| --- | --- | --- | --- | --- | --- |
| 18.1 | The risks to health and safety associated with any hazardous manual tasks have been identified and managed. Work Health and Safety Regulation 2011 s.60 |  |  |  |  |
| 18.2 | In determining what controls to implement to manage the risks associated with hazardous manual tasks, all relevant matters that may contribute to a musculoskeletal disorder, as described under the regulations, have been considered. Work Health and Safety Regulation 2011 s.60 |  |  |  |  |

#### 19. Hazardous substances and dangerous goods

| Ref | Criteria | Observations | Rating | Demerits | CAN details |
| --- | --- | --- | --- | --- | --- |
| 19.1 | The risks to health and safety associated with any hazardous chemicals used, handled, generated, or stored at the workplace have been identified and managed, including the risk: - of a physical or chemical reaction - of fire or explosion - that a chemical may become unstable. Work Health and Safety Regulation 2011 s.351, s.354, s.355 and s.356 |  |  |  |  |
| 19.2 | Controls implemented to eliminate or minimise the risks associated with hazardous chemicals are reviewed and revised in accordance with the regulations. Work Health and Safety Regulation 2011 s.352 |  |  |  |  |
| 19.3 | All hazardous chemicals that are used, handled, or stored at the workplace are correctly labelled. Work Health and Safety Regulation 2011 s.341 and s.342 | Note: The chemical classification and labelling provisions of the previous legislation (WHS Act 1995 and the Dangerous Goods Safety Management Act 2001) operates concurrently with the requirement of the new legislation to transition to the Globally Harmonised System of Classification and Labelling of Chemicals (GHS) by 31 December 2018. |  |  |  |
| 19.4 | A safety data sheet (SDS) exists and is available for inspection for each hazardous chemical used, handled, or stored at the workplace. Work Health and Safety Regulation 2011 s.344 |  |  |  |  |
| 19.5 | The relevant SDSs are available to: - workers using, handling, or storing a hazardous chemical - emergency service workers - any other person likely to be exposed to a hazardous chemical. Work Health and Safety Regulation 2011 s.344 |  |  |  |  |
| 19.6 | A register of hazardous chemicals used, handled, or stored at the workplace has been prepared in accordance with the regulations and is kept at the workplace. Work Health and Safety Regulation 2011 s.346 |  |  |  |  |
| 19.7 | A spill containment system has been implemented in each part of the workplace where there is a risk from a spill or leak of a hazardous chemical. Work Health and Safety Regulation 2011 s.357 |  |  |  |  |
| 19.8 | Fire protection and fire fighting equipment that has been designed and built for the types of hazardous chemicals at the workplace have been provided. Work Health and Safety Regulation 2011 s.359 |  |  |  |  |
| 19.9 | Emergency equipment is available at the workplace. Work Health and Safety Regulation 2011 s.360 |  |  |  |  |
| 19.10 | Appropriate levels and types of health monitoring is provided to workers that use, handle, generate, or store hazardous chemicals mentioned in schedule 14, table 14.1, column 2 of the regulation. Work Health and Safety Regulation 2011 s.368 and s.370 |  |  |  |  |
| 19.11 | Workers have been provided information about any health monitoring requirements. Work Health and Safety Regulation 2011 s.369 |  |  |  |  |
| 19.12 | The PCBU must ensure health monitoring is carried out in accordance with the regulations, and: - is conducted under the supervision of a registered medical practitioner with the relevant experience - is paid for by the relevant PCBUs - a report provided to the relevant worker - a report provided to the regulator, if required - appropriate records are kept. Work Health and Safety Regulation 2011 s.371, s.372, s.375, s.376, and s.378 |  |  |  |  |
| 19.13 | Appropriate supervision is provided to workers in relation to hazardous chemicals. Work Health and Safety Regulation 2011 s.379 |  |  |  |  |

#### 20. Asbestos management

| Ref | Criteria | Observations | Rating | Demerits | CAN details |
| --- | --- | --- | --- | --- | --- |
| 20.1 | A SWMS is available for all work that involves or is likely to involve the disturbance of asbestos that has commenced or is about to commence. Work Health and Safety Regulation 2011 s.291 and s.299 |  |  |  |  |
| 20.2 | Exposure of a person at the workplace to airborne asbestos has been eliminated or, where elimination s not reasonably practicable, minimised so far as is reasonably practicable. Work Health and Safety Regulation 2011 s.420 |  |  |  |  |
| 20.3 | The exposure standard for asbestos has not been exceeded at the workplace. Work Health and Safety Regulation 2011 s.420 |  |  |  |  |
| 20.4 | All asbestos or ACM present at the workplace has been identified by a competent person. Work Health and Safety Regulation 2011 s.422 |  |  |  |  |
| 20.5 | The presence and location of asbestos or asbestos containing material is clearly indicated. Work Health and Safety Regulation 2011 s.424 |  |  |  |  |
| 20.6 | If required, an asbestos register is: - prepared and kept at the workplace - reviewed and revised in accordance with the regulations - accessible to the relevant persons. Work Health and Safety Regulation 2011 s.425, s.426, and s.427 | Note: For workplace buildings, an asbestos register is not required if it meets all of the following requirements. - The building was constructed after 31 December 1989. - No asbestos has been identified at the workplace. - No asbestos is likely to be present at the workplace from time to time. |  |  |  |
| 20.7 | If required, an asbestos management plan is: - prepared and maintained for the workplace - reviewed and revised in accordance with the regulations - accessible to the relevant persons. Work Health and Safety Regulation 2011 s.429 |  |  |  |  |
| 20.8 | Workers involved in asbestos removal work or in the carrying out of asbestos-related work have been trained in the identification of and safe handling of asbestos and ACM. Work Health and Safety Regulation 2011 s.445 |  |  |  |  |
| 20.10 | Asbestos removal work is carried out by an appropriately licensed removalist. Work Health and Safety Regulation 2011 s.458 |  |  |  |  |
| 20.11 | Asbestos removal work is carried out in accordance with the regulations. Work Health and Safety Regulation 2011 Part 8.7 Asbestos removal work |  |  |  |  |

#### 21. Noise

| Ref | Criteria | Observations | Rating | Demerits | CAN details |
| --- | --- | --- | --- | --- | --- |
| 21.1 | The risks to health and safety associated with noise at the workplace have been identified and managed. Work Health and Safety Regulation 2011 s.57 |  |  |  |  |
| 21.2 | Workers are not exposed to noise in excess of the exposure standard for noise Work Health and Safety Regulation 2011 s.57 |  |  |  |  |

#### 22. General traffic management

| Ref | Criteria | Observations | Rating | Demerits | CAN details |
| --- | --- | --- | --- | --- | --- |
| 22.1 | The principal contractor has managed the risks to health and safety associated with traffic in the vicinity of the workplace that may be affected by construction work carried out in connection with the construction project. Work Health and Safety Regulation 2011 s.315 |  |  |  |  |
| 22.2 | A SWMS is available for all work that is carried out on, in, or adjacent to a: - road - railway - shipping lane - other traffic corridor that is in use by traffic (other than pedestrians) that has commenced or is about to commence. Work Health and Safety Regulation 2011 s.291 and s.299 |  |  |  |  |

#### 23. Hazardous atmosphere

| Ref | Criteria | Observations | Rating | Demerits | CAN details |
| --- | --- | --- | --- | --- | --- |
| 24.1 | A SWMS is available for all work that is carried out in an area that may have a contaminated or flammable atmosphere that has commenced or is about to commence. Work Health and Safety Regulation 2011 s.291 and s.299 |  |  |  |  |
| 24.2 | The risks to health and safety associated with hazardous atmospheres have been identified and managed. Work Health and Safety Regulation 2011 s.51 |  |  |  |  |

#### 25. Other hazards and risks

| Ref | Criteria | Observations | Rating | Demerits | CAN details |
| --- | --- | --- | --- | --- | --- |